🧪 FRAP-Tracker ADCANCED by IGB

Advanced FRAP Analysis with Independent Tracking of Both Nucleus and Foci (2D Version)

https://img.shields.io/badge/python-3.7+-blue.svg
https://img.shields.io/badge/License-MIT-yellow.svg

📋 Description

FRAP-Tracker ADCANCED is an advanced Python-based tool for analyzing FRAP (Fluorescence Recovery After Photobleaching) experiments in 2D time-lapse images. This version represents a significant evolution from the BASIC version, addressing more complex experimental scenarios where both the nucleus and the foci can move independently during the experiment.

Key Features:

- 🎯 Dual independent tracking - Both nucleus and foci tracked separately

- 🔄 Intensity-based foci tracking - Finds brightest spot in search area

- 🔍 Phase correlation nucleus tracking - Robust displacement detection

- 📊 EasyFRAP-compatible CSV output with all three intensities

- 🖼️ Frame-by-frame visualization with color-coded ROIs

- 📈 Comprehensive analysis with FRAP recovery calculation

- 🖱️ Semi-automatic workflow with user validation at key steps

## 🚀 Installation

### Prerequisites

- Python 3.7 or higher

- pip package manager

### Install from source

bash

# Clone the repository

git clone https://github.com/yourusername/frap-tracker-advanced.git

cd frap-tracker-advanced

# Install required packages

pip install -r requirements.txt

### Required Packages

Create a requirements.txt file with:

text

numpy>=1.19.0

pandas>=1.2.0

scikit-image>=0.18.0

scipy>=1.6.0

matplotlib>=3.3.0

pillow>=8.0.0

tkinter (included with Python)

## 📖 How It Works

### Core Algorithm

#### Part 1: Initialization Phase

- Load MASK image - Automatic binarization using Otsu thresholding

- Detect initial ROI - Finds largest contour for reference

- Select FOCI ROI on first PRE frame (red circle):

  - User clicks center, adjusts radius (1-20 px)

  - Saved as reference for PRE bleach series

- Manually trace NUCLEUS ROI on BLEACH frame (green polygon):

  - Minimum 5 points for accurate contour

  - Program calculates geometric center and area

- Select BACKGROUND ROI (blue polygon):

  - Slider to view all frames (PRE, BLEACH, POST)

  - Area WITHOUT cells - static reference

- Select FOCI position on BLEACH frame:

  - Starting point for POST-bleach tracking

#### Part 2: Independent Tracking Across All Frames

Key principle: Each structure tracked separately with different algorithms:

| Structure | Tracking Method | Search Strategy |
| --- | --- | --- |
| NUCLEUS | Phase correlation | Whole-frame displacement detection |
| FOCI | Maximum intensity | Local search within radius |
| BACKGROUND | Static | Fixed polygon, no tracking |

Tracking workflow:

- PRE BLEACH frames (forward tracking):

  - Start from user-selected foci on first PRE frame

  - Track forward through all PRE frames

  - Nucleus tracked via correlation with BLEACH reference

- BLEACH frame (t=0):

  - Uses user-selected foci position

  - Reference for POST-bleach tracking

- POST BLEACH frames (forward tracking):

  - Start from BLEACH foci position

  - Sequential tracking through all POST frames

  - Nucleus continuously updated

#### Time Scale

- BLEACH frame = 0 seconds

- PRE frames = negative time (before bleach)

- POST frames = positive time (after bleach)

### Output Files

CSV file (frap_analysis_results_2D.csv):

text

Time_s,FOCI_Intensity,Nucleus_Intensity,Background_Intensity

-10.0,245.3,187.2,45.1

-5.0,243.8,186.5,45.3

0.0,89.4,185.6,45.1

5.0,156.7,184.9,45.2

10.0,189.2,185.1,45.0

Results folder structure:

text

FRAP_Analysis_2D_YYYYMMDD_HHMMSS/

├── frap_analysis_results_2D.csv

├── intensity_measurements.csv

├── analysis_report_2D.txt

├── algorithm_description.txt

└── processed_frames/

├── frame_0000.png

├── frame_0001.png

└── ...

## 🖱️ User Guide

### Step-by-Step Instructions

#### 1. Launch the Program

bash

python frap_tracker_advanced.py

#### 2. Enter Parameters

- Resolution - Pixels per micron (e.g., 6.5)

- Search radius - Max foci movement in microns (typically 1-5)

- Time interval - Seconds between frames (e.g., 5.0)

#### 3. Select Image Files

Sequentially select (in this order):

- PRE_BLEACH - Images before bleaching (single or multi-page TIFF)

- BLEACH - Image at bleach moment

- POST_BLEACH - Images after bleaching

- MASK_BLEACH - Binary mask or image for ROI detection

#### 4. Validate Mask Binarization

- Program shows original and binarized mask

- Click "Yes" if correct, "No" for manual override (placeholder)

#### 5. Select FOCI ROI on First PRE Frame

- LEFT PANEL (PRE): Click on foci center (red circle appears)

- Adjust radius with bottom slider (1-20 pixels)

- RIGHT PANEL (BLEACH): Shown for reference only

- Click "✅ Finish selection"

#### 6. Manually Trace NUCLEUS on BLEACH Frame

- Left-click to trace nucleus outline (minimum 5 points)

- Click "❌ Remove point" to delete last point

- Click "✅ FINISH SELECTION" to complete

#### 7. Select BACKGROUND ROI

- Use slider to view all frames (PRE, BLEACH, POST)

- Left-click to trace area WITHOUT cells (minimum 3 points)

- Click "✅ Finish selection"

#### 8. Select FOCI Position on BLEACH Frame

- Use slider to find best BLEACH frame

- Left-click on foci center (red circle with radius)

- Click "✅ Finish selection"

#### 9. Choose Output Folder

- Select folder for results

- If cancelled, creates FRAP_Tracker_Advanced_Results_2D in image directory

#### 10. Final Verification

- Program shows three panels with TRACKED ROIs:

  - PRE BLEACH (first frame)

  - BLEACH (bleach moment)

  - POST BLEACH (last frame)

- Verify all ROIs correctly placed

- Click "✅ YES, EVERYTHING IS CORRECT" or "❌ NO, REDO SELECTION"

## 🎨 Visualization Color Scheme

| Color | ROI | Description |
| --- | --- | --- |
| 🔴 RED | FOCI | Intensity measurement region - tracked independently |
| 🟡 YELLOW | Search area | Where algorithm looks for foci in next frame |
| 🟢 GREEN | NUCLEUS | Tracked nucleus contour with center marker |
| 🔵 BLUE | BACKGROUND | Static background measurement region |

## 📊 Understanding Results

### Intensity Measurements

Three intensity values are reported for each frame:

- FOCI_Intensity - Mean intensity in red circle (tracked)

- Nucleus_Intensity - Mean intensity within green contour (tracked)

- Background_Intensity - Mean intensity in blue polygon (static)

### FRAP Recovery Calculation

The analysis report automatically calculates:

- Average PRE-bleach intensity

- BLEACH intensity (minimum)

- Maximum POST-bleach recovery

- Recovery percentage: (max_post - bleach) / (avg_pre - bleach) * 100%

### Quality Control

Successful tracking indicators:

- Smooth intensity curves without sudden jumps

- Logical movement patterns (gradual, within search radius)

- Consistent nucleus area (automatically corrected ±10%)

## ⚠️ Important Notes

- Independent tracking means foci can move relative to nucleus

- Search radius critical parameter - too small loses foci, too large causes false positives

- Nucleus area preserved across frames (±10% tolerance)

- Background ROI is static - choose area without any movement

- PRE frames tracked forward from first frame

- If tracking fails, previous position used (warning logged)

## 🔧 Troubleshooting

| Problem | Solution |
| --- | --- |
| Foci lost during tracking | Increase search radius |
| Nucleus area changes drastically | Check image quality, reselect nucleus with more points |
| Background intensity fluctuates | Choose more uniform area without cells |
| Tracking fails on PRE frames | Start with clearer first PRE frame |
| Program hangs on image load | Check TIFF format, file integrity |
| Recovery calculation shows >100% | Normal if pre-bleach had bleaching |
| CSV has duplicate times | Program auto-corrects with epsilon |

## 📄 Advanced Algorithm Details

### FOCI Tracking Algorithm

python

1. Start from previous frame's foci position

2. Define circular search area (radius = user parameter in microns)

3. Scan all pixels within search area with step=2

4. For each candidate center, calculate mean intensity in foci ROI

5. Select position with MAXIMUM mean intensity

6. Update foci position for next frame

### Nucleus Tracking Algorithm

python

1. Use BLEACH frame as reference

2. Calculate phase correlation between reference and current frame

3. Find peak correlation to determine displacement

4. Shift nucleus mask by displacement vector

5. Correct area if changed >10% (dilate/erode)

6. Update nucleus contour and center

### Why Independent Tracking?

The ADVANCED version is essential when:

- FOCI moves within the nucleus (e.g., chromatin dynamics)

- Nuclear shape changes but foci position shifts independently

- Cells rotate, causing foci to move relative to nucleus

- Maximum intensity gives better foci localization than fixed offset

## 📁 File Format Support

- TIFF - Single images or multi-page stacks (recommended)

- PNG/JPG - Single images only

- All formats - Automatically converted to grayscale

## 📄 License

MIT License - feel free to use, modify, and distribute

## 👥 Authors

Developed by Ksenia Kudryashova at IGB (Institute of Gene Biology)

## 📧 Contact

For questions, suggestions, or bug reports, please open an issue on GitHub.

## 🙏 Acknowledgments

- Beta testers for valuable feedback

- Built with scikit-image, matplotlib, and numpy

- Inspired by real FRAP experiment needs with moving cells

- EasyFRAP-compatible output format for further analysis

Version: 2.0 | Last Updated: March 2026 | Status: Production Ready

For BASIC version (fixed foci relative to nucleus), see FRAP-Tracker BASIC by IGB