# 🧪 FRAP-Tracker BASIC by IGB

Automated FRAP Analysis with Nucleus Tracking for Moving Cells

https://img.shields.io/badge/python-3.7+-blue.svg
https://img.shields.io/badge/License-MIT-yellow.svg

## 📋 Description

FRAP-Tracker BASIC is a Python-based tool for analyzing FRAP (Fluorescence Recovery After Photobleaching) experiments where cellular structures (nucleus and foci) may move between frames. The program features automatic nucleus tracking using phase correlation and calculates foci position relative to the nucleus, eliminating the need for manual ROI selection on each frame.

Key Features:

- 🔄 Automatic nucleus tracking across PRE, BLEACH, and POST frames

- 🎯 Fixed relative positioning of foci to nucleus

- 📊 EasyFRAP-compatible CSV output

- 🖼️ Frame-by-frame visualization with overlaid ROIs

- 📈 Built-in intensity and movement graphs

- 🖱️ User-friendly interactive ROI selection

## 🚀 Installation

### Prerequisites

- Python 3.7 or higher

- pip package manager

### Install from source

bash

# Clone the repository

git clone https://github.com/yourusername/frap-tracker-basic.git

cd frap-tracker-basic

# Install required packages

pip install -r requirements.txt

Required Packages

Create a requirements.txt file with:

text

numpy>=1.19.0

pandas>=1.2.0

scikit-image>=0.18.0

scipy>=1.6.0

matplotlib>=3.3.0

pillow>=8.0.0

📖 How It Works

Core Algorithm

Part 1: Initialization on BLEACH Frame

- User selects FOCI ROI (red circle) - click center, adjust radius (1-20 px)

- User traces NUCLEUS ROI (green polygon) - minimum 5 points

- Program calculates geometric nucleus center (intensity-independent)

- Saves relative foci position vector: foci_offset = foci_center - nucleus_center

- BACKGROUND ROI selected with slider across all frames

Part 2: Frame Processing & Tracking

Key principle: Foci position is always calculated as: foci_position = current_nucleus_center + saved_offset_vector

Time scale:

- BLEACH frame = 0 seconds

- PRE frames = negative time (before bleach)

- POST frames = positive time (after bleach)

Tracking method: Phase correlation for robust displacement detection

Processing order:

- PRE frames: reverse order (last → first) for better accuracy

- POST frames: forward order (first → last)

- Each frame uses previous frame as reference for tracking

Output Files

CSV file (forEasyFRAP_experimentname.csv):

text

Channel.001

Axis [s],ROI1 [],ROI2 [],ROI3 []

0.0,245.3,187.2,45.1

5.0,189.4,185.6,45.3

...

Results folder structure:

text

FRAP_Analysis_YYYYMMDD_HHMMSS/

├── forEasyFRAP_experimentname.csv

├── Parameters.txt

├── Processed_Frames/

│   ├── frame_pre_0_time_-10.0s.png

│   ├── frame_bleach_0_time_0.0s.png

│   └── frame_post_0_time_5.0s.png

🖱️ User Guide

Step-by-Step Instructions

1. Launch the Program

bash

python frap_tracker_basic.py

2. Enter Experiment Name

- Enter a descriptive name (letters, numbers, _, -)

- Used in output filename: forEasyFRAP_name.csv

3. Select Image Files

Sequentially select:

- PRE_BLEACH - images before bleaching

- BLEACH - image at bleach moment

- POST_BLEACH - images after bleaching

Supports single images or multi-page TIFF stacks

4. Enter Frame Interval

- Time between frames in seconds (e.g., 5.0)

5. Select FOCI ROI on BLEACH Frame

- Left-click on foci center (red circle appears)

- Adjust radius with bottom slider (1-20 pixels)

- Click "✅ Finish"

6. Select NUCLEUS ROI on BLEACH Frame

- Left-click to trace nucleus outline (minimum 5 points)

- Click "❌ Remove point" to delete last point

- Click "✅ FINISH" to complete

7. Select BACKGROUND ROI

- Use slider to view all frames (PRE, BLEACH, POST)

- Left-click to trace area WITHOUT cells (minimum 3 points)

- Click "✅ Finish selection"

8. Choose Output Folder

- Select folder for results

- If cancelled, creates FRAP_Analysis_Results in current directory

9. View Results

- Intensity plots and nucleus movement graphs open automatically

- Check for successful tracking (smooth curves, logical movement)

📊 Understanding Results

Intensity Plot

- Red line - Foci intensity (bleached region)

- Green line - Nucleus intensity (control)

- Blue line - Background intensity

- Dashed vertical line - Bleach moment (t=0)

Nucleus Movement Plot

- Shows X and Y coordinates of geometric center over time

- Large displacements (> foci radius) may indicate unreliable results

CSV File

- Ready for EasyFRAP, Prism, Excel, or any plotting software

- Time normalized to start at 0 seconds

- Columns: time, foci intensity, nucleus intensity, background intensity

⚠️ Important Notes

- No direct foci tracking - position calculated from nucleus + saved offset

- Geometric center used instead of intensity-based center for robustness

- PRE frames processed in reverse for better accuracy with movement

- If tracking fails, program uses last known coordinates (with warning)

- All frames with overlaid contours saved for quality control

🔧 Troubleshooting

| Problem | Solution |
| --- | --- |
| Tracking fails | Ensure sufficient contrast, select more nucleus points |
| Foci radius wrong | Adjust during selection, can't be changed later |
| Duplicate times in CSV | Program auto-corrects with small epsilon |
| Images not loading | Check file format (TIFF only) |

📄 License

MIT License - feel free to use, modify, and distribute

👥 Authors

Developed by Ksenia Kudryashova at IGB (Institute of Gene Biology)

📧 Contact

For questions or suggestions, please open an issue on GitHub.

🙏 Acknowledgments

- Thanks to all beta testers

- Built with scikit-image, matplotlib, and numpy

- EasyFRAP-compatible output format